IBM 3174 Establishment Controller

The IBM 3174 Establishment Controller was developed at the IBM Development Laboratory, Research Triangle Park, North Carolina and was announced in 1986.  It was a replacement for the IBM 3274 cluster controller.

3270 terminals were physically attached to the 3174 via a coaxial cable; dumb devices (e.g. 3278, 3279, etc.) using CUT (Control Unit Terminal) protocol.  Intelligent devices (e.g. 3270PC) using DFT (Distributed Function Terminal) protocol.  A feature also allowed Asynchronous ASCII terminals to be attached.  Later, downstream devices could be attached via Token Ring Local Area Network.

3174 can be attached to a host computer either locally (“L” Models) via a System/370 Channel connection or remotely (R Models) via modems to V.24, V35 or X.21 Interfaces.  Later Models could be attached via a Token Ring Local Area Network.