Power on Self-Test codes

When power is first applied, initial self tests are performed by

built it ROM routines and special processor on the system planar

board.  These self tests should complete with in 1-2 minutes.

The tests labelled 'fatal' indicate that if this code remains for

more than five minutes, indicates that component has failed.

Note: I have seen tests 14,15,16 and 17  last for up to three to four

minutes, depending on the number of hard disks and/or floppy drives

connected to the installed adapters.

ROS LED Values during IPL

00      Initialization of Mono/Printer adapter failed

01      ROS CRC did not compare - Fatal

02      Soft IPL check failed - Fatal

03      Memory error or no memory - Fatal

04      Processor card logic error - Fatal

05      Processor or memory error condition - Fatal

07      IOCC test resident POST - Fatal

08      Bad processor card in IOCC test - Fatal

09      Keyboard adapter resident POST - Fatal

0c      initialization failed - Mono/Printer adapter and APC

10      System timer resident POST - Fatal

11      Interrupt controller POST - Fatal

12      DMA arbiter resident POST - Fatal

13      Serial port resident POST

14      Fixed disk resident POST adapter 1 test

15      Fixed disk resident POST adapter 2 test

16      Diskette resident POST adapter 1 test

17      Diskette resident POST adapter 2 test

18      Extension ROS

19      Attempting Manufacturing IPL

1c      Advanced Processor card - Fatal

1u      Enhanced Advanced Processor and memory Mgmt card - Fatal

20      NVRAM CRC check

21      No boot record found (NVRAM selected devices)

22      No boot record found (ROS selected devices)

23      Disk or diskette adapter slot or address - Fatal

25      User error - invalid RamSpecReg - Fatal

26      Attempting soft IPL

27      Bootable code exceeds available storage - Fatal

28      Unexpected return from loaded code - Fatal

29      IPL process passed control to loaded code.

3c      Advanced Processor card and first 128K of memory.

3u      Enhanced Advanced Processor and memory Mgmt card.

4c      APC logic (Advanced Processor Card).

4u      Enhanced Advanced Processor and memory Mgmt card Logic.

5c      APC logic and system memory.

5u      Memory expansion option or Enhanced Advanced Processor and

memory management card.

88      reserved - POR or processor check

89      Unexpected machine or program check - Fatal.

8c      IOCC test for APC.

8u      Enhanced Advanced processor and memory management card.

96      Memory card 1 resident POST error condition.

97      Memory card 2 resident POST error condition.

98      Memory card 1 and 2 resident POST error condition.

99      KEY locked, locked response.