IBM 7094

The 7094 was IBM’s most powerful scientific computer in 1963. An “upgrade” from the IBM 7090, the 7094 was 1.4 to 2.4 times faster than its predecessor  it could perform 500,000 logical decisions, 250,000 additions or subtractions, 100,000 multiplications, or 62,500 divisions in one second. It had hardware to do double-precision floating-point arithmetic.

The computer gained considerable I/O bandwidth from its separate data channels with direct memory access, and so it was also used to run business and general-purpose applications. The 7094 had an operating system called IBSYS, and FORTRAN and COBOL compilers.

A typical system cost $3,134,500. IBM stopped selling them in 1969.

Sources: C. Gordon Bell, et al. Computer Structures: Readings and Examples. New York: McGraw-Hill, 1971. pp. 515-523

Manufacturer:	IBM	Memory technology:	magnetic core

First introduced:	1963	Memory size:	32K 36-bit words

CPU technology:	transistor	Cycle time:	2 microseconds (0.5 MHz)

Charles Basche, et al. IBM’s Early Computers. Cambridge, MA: MIT Press, 1986. p. 449

IBM 7094 Principles of Operation. IBM Systems Reference Library, 1962

IBM 7090/7094 IBSYS Operating System. IBM Systems Reference Library, 1964

IBM 7090/7094 IBSYS Operating System Version 13 FORTRAN IV Language. IBM Systems Reference Library, 1968

http://www.multicians.org/thvv/7094.html

http://www.multicians.org/mga.html

http://www.ibm.com/news/ls/1999/07/articles/sidebar_13.phtml

Photo by IBM (http://www.ibm.com/news/ls/1999/07/photoarchive/photoarchive_13.phtml)